| COURSE | Network Fundamentals | ASSIGNMENT NO |  |
| --- | --- | --- | --- |
| MODULE | Final Assessment | ASSIGNMENT DATE | 6/09/2024 |
| STUDENT NAME | Konganti Chaithanya Kumar | SUBMISSION DATE | 8/09/2024 |

LAB 2

----------------------------------------------------------------------

- Use the following network, select a Network ID of your choice and do subnetting to assign the required subnets and host IP’s in each subnet.

- Do either static/OSPF routing and establish the connectivity.

- Now permit one PC from Subnet 1 to access second subnet and server 1 not server 2, access any PC from second subnet 2 to server 2 through HTTP only and not server 1, deny ping from subnet 2 to both servers.

Solution:

We have chosen the network 192.168.20.0/24 and need to create 5 subnets. To achieve this, we'll subnet the network to accommodate the required subnets.

Subnetting Steps:

- Original Network: 192.168.20.0/24 (provides 256 IP addresses)

- Required Subnets: 5

Subnet Calculation:

- Subnet 1: 192.168.20.0/27 (Usable IPs: 192.168.20.1 - 192.168.20.30)

- Subnet 2: 192.168.20.32/27 (Usable IPs: 192.168.20.33 - 192.168.20.62)

- Subnet 3: 192.168.20.64/27 (Usable IPs: 192.168.20.65 - 192.168.20.94)

- Subnet 4: 192.168.20.96/27 (Usable IPs: 192.168.20.97 - 192.168.20.126)

- Subnet 5: 192.168.20.128/27 (Usable IPs: 192.168.20.129 - 192.168.20.158

### 1. Setting Up the Network Topology

- Add Routers to the Workspace:

- Connect the Routers:

    - Router 1 to Router 2

    - Router 2 to Router 3

Router 1:

- Interface GigabitEthernet0/0: 192.168.20.1/27

- Interface GigabitEthernet0/1: 192.168.20.97/27

Router 2:

- Interface FastEthernet0/0: 192.168.20.98/27

- Interface FastEthernet0/1: 192.168.20.33/27

- Interface Ethernet1/0: 192.168.20.129/27

Router 3:

- Interface GigabitEthernet0/0: 192.168.20.130/27

- Interface GigabitEthernet0/1: 192.168.20.65/27

- Add PCs/Laptops to the Workspace

Use appropriate cables (e.g., straight-through or crossover) to connect each PC or laptop to the router or switch in the workspace.

Topology Diagram:

- Configuring IP Address, Subnet Mask, and Default Gateway for Devices:

.

IP Address for subnet 1:

IP Address for subnet 2:

IP Address for subnet 3:

2. Do static routing and establish the connectivity.

### Router 1 Configuration

### Router 2 Configuration

### Router 3 Configuration

Ping and check the connectivity between different networks:

Now permit one PC from Subnet 1 to access second subnet and server 1 not server 2, access any PC from second subnet 2 to server 2 through HTTP only and not server 1, deny ping from subnet 2 to both servers.

### Conditions to Apply

- Permit one PC from Subnet 1 to access Subnet 2 and Server 1 but not Server 2.

- Allow any PC from Subnet 2 to access Server 2 via HTTP only, and not Server 1.

- Deny ping (ICMP) from Subnet 2 to both servers.

### Steps to Apply Rules on Each Router :

#### Router 1 (R1)

Objective: Allow traffic from Subnet 1 to Subnet 2 and Server 1 but not to Server 2.

- Create and Apply Access Control List (ACL) on Router 1:

  - Create ACL to Allow Access for any one PC to Subnet 2.

R3(config)#ip access-list extended 100

R3(config-ext-nacl)# permit ip host 192.168.20.3 192.168.20.32 0.0.0.31

R3(config-ext-nacl)# permit ip host 192.168.20.3 host 192.168.20.66

This ACL allows the PC with IP address 192.168.20.3 in Subnet 1 to access devices in Subnet 2 and Server 1(in Subnet3) while blocking access to Server 2.

  - Apply the ACL to the interface connecting to Subnet 1:

R2(config)#interface GigabitEthernet0/0

R2(config-if)#ip access-group 100 in

#### Router 2 (R2)

Objective: Allow any PC from Subnet 2 to access Server 2 via HTTP only, and deny ICMP traffic from Subnet 2 to both servers.

- Create and Apply Access Control List (ACL) on Router 2:

R3(config)#ip access-list extended 100

R3(config-ext-nacl)# permit ip host 192.168.20.32 0.0.0.31 host 192.168.20.67 eq www

R3(config-ext-nacl)# deny icmp 192.168.20.32 0.0.0.31 host 192.168.20.66

R3(config-ext-nacl)# deny icmp 192.168.20.32 0.0.0.31 host 192.168.20.67

The configuration allows any PC from Subnet 2 to access Server 2 via HTTP only, while ICMP traffic from Subnet 2 to both servers is denied.

- Apply the ACL to the interface connecting to Subnet 2:

R2(config)#interface FastEthernet 0/1

R2(config-if)#ip access-group 100 in

ACL Configuration Results showing ACL configuration correctly permits only HTTP access to Server 2

The result showing ACL configuration correctly denies ping requests from Subnet 2 to both servers.